CONTRACT NO. XX-DSR-XXXXX

SCHEDULING, ACCOUNTING AND BILLING PROCEDURES

BETWEEN

UNITED STATES

DEPARTMENT OF ENERGY

WESTERN AREA POWER ADMINISTRATION

Desert Southwest Customer Service Region

Parker-Davis Project

AND

XXXX

Table of Contents

Section	Title	Page No.

### 1	Parties		1

2	Purpose		2

3	Term		2

4	Revision		2

5	Point(s) of Delivery		2

6	Definitions		2

7	P-DP Application		2

8	Energy		2

9	General Scheduling Guidelines		4

10	Hourly CROD		7

11	General Transmission 		8

12	Seasonal Full Transmission Usage for COCP		8

13	Power Accounting and Billing Procedures		9

Attachment No. 1, Contractor’s Scheduling Entity and Point of Contacts (Pending Development)

Attachment No. 2, WAPA’s Point of Contacts (Pending Development)

CONTRACT NO. XX-DSR-XXXXX

SCHEDULING ACCOUNTING AND BILLING PROCEDURES

BETWEEN

UNITED STATES

DEPARTMENT OF ENERGY

WESTERN AREA POWER ADMINISTRATION

Desert Southwest Customer Service Region Office

Parker-Davis Project

AND

XXXX

# 1.	PARTIES:  The Parties to these Scheduling, Accounting and Billing Procedures (SABP) are Western Area Power Administration (WAPA) and XXXX (Contractor), each sometimes individually called Party and collectively called Parties.

# Authorized Representative:  Contractor will designate their Authorized Representative(s) in writing or electronically through the Parker-Davis Project

# (P-DP) application on WAPA’s portal (P-DP Application) in accordance with Section 27 of the Contract.  If designation is done in writing WAPA will update the P-DP Application with the new information.

# Scheduling Entity:  Contractor or Authorized Representative will designate Contractor’s Scheduling Entity(ies) (SE) in writing or electronically through the P-DP Application in accordance with Section 15 of the Contract.

# Contractor or Authorized Representative may request changes to Contractor’s Scheduling Entity(ies) (SE) in accordance with Section 15 of the Contract.   Electronic requests will be made through the P-DP Application and WAPA will approve in the application.  If the request is a written request, WAPA will update the P-DP Application once the new SE is approved.

# A list of all WAPA Point of Contacts and permissions will be provided in the P-DP Application.

# 2.	PURPOSE:  This SABP is written to implement the scheduling, accounting and billing contractual requirements in accordance with the applicable Sections of the Contract.  If there are any conflicts between the terms of the Contract and the terms of this SABP, the terms of the Contract shall prevail.

# 3.	TERM:  This SABP shall become effective in accordance with Section 4 of the Contract and will remain in effect until superseded by revised procedures in accordance with Section 17 of the Contract or termination of the Contract.

# 4.	REVISION:  The Parties intend that this SABP shall be reviewed periodically, and 	revised as necessary in accordance with Section 17 of the Contract.

# 5.	INTENTIONALLY OMITTED

- DEFINITIONS:  Terms, when used herein and in the Attachments, hereto, whether in singular or plural, shall have the meanings set forth in Section 5 (Definitions) of the Contract.

- P-DP APPLICATION:  The P-DP Application will be located on WAPA’s Portal.  The application is intended to streamline processes by housing information in one location and allows Contractors direct access to the following:

  - Quarterly Energy and Excess Energy monthly amounts.

  - Quarterly Energy reductions when applicable.

  - Estimated Optional Energy pricing.

  - Elections of Optional Energy.

  - Elections of Seasonal Full Transmission Usage.

  - Request changes to authorized representatives and Scheduling Entities.

  - Minimum requirements.

  - Hourly CROD available to schedule.

  - Notifications as applicable.

# WAPA will engage the Contractor and provide instructions on the use of the P-DP    Application.

- ENERGY:  Energy is comprised of the following components:

8.1	Quarterly Energy:  The Contractor’s Quarterly Energy will be based on Reclamation’s Most Probable energy forecast in the hydro power generation projection studies, which are updated monthly.

8.1.1	Quarterly Energy will be viewable on the P-DP Application for Contractors by no later than the last day of August for October through December, the last day of November for January through March, the last day of February for April through June, and the last day of May for July through September, of each year of the Contract.

8.1.2	WAPA will publish on the P-DP Application, the full megawatt (MW) CROD aggregated by Scheduling Entity, prior to the start of the month for rounding purposes.

8.1.3	Contractor’s Quarterly Energy will be rounded to the nearest whole megawatt hour and scheduled in accordance with Section 9 herein.

8.1.4	WAPA will post to the P-DP Application the Contractor’s monthly minimum schedule for each hour for Quarterly Energy prior to the beginning of the month, based on hourly power system and water release requirements, in accordance with Section 10 of the Contract.

8.1.4.1  WAPA will calculate minimum values for Quarterly Energy that   the Contractor must take in certain hours of the month.

8.1.4.2  Minimum values for Quarterly Energy will be calculated with a Minimum Schedule Requirement (MSR) model.  The MSR model follows the following steps:

8.1.4.2.a	Uses historical market data to identify the lowest-	priced hours.

8.1.4.2.b	Identifies which of those low-priced hours have historically had more generation than load and extends the requirement to any hour priced lower than the lowest identified hour.

8.1.4.2.c	Allocates minimum scheduled energy across 	Contractors by CROD share and hourly generation 	percentages.

8.1.5	In the event of a reduction in Quarterly Energy made by WAPA in accordance with subsection 8.1.3 of the Contract, the revised Quarterly Energy would be effective no later than two days prior to day-ahead prescheduling.  WAPA will post revised Quarterly Energy to the P-DP Application and provide email notification through the P-DP Application.

8.2	Optional Energy:  Contractor may request WAPA to purchase Optional Energy to maximize CROD or to meet any deficiency in Quarterly Energy in accordance with subsection 8.1.3 and Section 8.2 of the Contract.

8.2.1	Contractors electing Optional Energy will make their election in the P-DP Application X days prior to the beginning of each month.  Optional Energy shall be scheduled no later than one day prior to day-ahead prescheduling.  Contractors electing Optional Energy must advance funds to cover the estimated cost of Optional Energy.  Billing and accounting of Optional Energy is in accordance with Subsection 13.3, herein.

8.2.2	In the event of a reduction in accordance with Subsection 8.1.3 of the Contract, Contractor may request Optional Energy through the P-DP Application upon notification of the reduction.

8.3	Excess Energy:  WAPA will designate Excess Energy using the formulas in Subsection 8.3.1, herein.

8.3.1	Excess Energy calculation formula:

Excess Energy = Projected annual generation kWh minus annual PUP kWh minus maximum Quarterly Energy kWh.

Maximum Quarterly Energy kWh = [FES summer marketable capacity kW x 214 (summer days) x 24 (hours per day) x 0.67 (summer capacity factor)] plus [FES winter marketable capacity kW x 151 (winter days) x 24 (hours per day) x 0.47 (winter capacity factor)]. Currently maximum Quarterly Energy equates to 904,863,151 kWh in summer, plus 344,210,745 kWh in winter, for a total of 1,249,073,896 kWh annually.

8.3.2	If Excess Energy is deemed to be available it will be issued at the beginning of the quarter along with Quarterly Energy.

8.3.3	Excess Energy will be subject to the same scheduling guidelines as Quarterly Energy.

8.3.4	Excess Energy will be billed in accordance with Subsection 13.2.2, herein.

8.4	Contractor Owned or Contractor Purchased Energy (COCP):  As defined in Section 5.9 of the Contract.

8.4.1	Transmission usage for COCP is explained in Subsections 11.3, 11.4, 11.5 and 12.1, herein.

8.4.2	Scheduling of COCP will be subject to the general scheduling guidelines and e-Tag requirements as set forth in Subsection 9.3, herein.

- GENERAL SCHEDULING GUIDELINES:

# 9.1	Scheduling:  Schedules associated with CROD and energy deliveries will be established and confirmed in Mountain Standard Time (MST) in advance as outlined in this SABP.

## 9.1.1	The Scheduling Entity is designated to implement schedules with WAPA on behalf of Contractor.  Each Party shall provide its scheduling points of contact and other information for this SABP in accordance with Subsection 1.2, herein.

## 9.1.2	The Scheduling Entity will schedule and WAPA will deliver Contractor's Quarterly Energy, Excess Energy, Optional Energy, and COCP in accordance with the Contract and this SABP.

## 9.1.3	CROD is listed in Exhibit A of the Contract and will also be available on the P-DP Application.  Quarterly Energy and Excess Energy amounts will be listed on the P-DP Application in accordance with Subsection 8.1. and 8.3, herein.

## 9.1.4	Scheduling practices will comply with applicable Western Area Lower Colorado (WALC) Business Practices, current North American Electric Standards Board (NAESB), North American Electric Reliability (NERC), Western Electricity Coordinating Council (WECC), and applicable Federal Energy Regulatory Commission (FERC), or successor(s), requirements.  All scheduling shall be statically scheduled and shall follow the Electronic Tagging (e-Tag) requirements in accordance with Subsections 9.1.4.1, 9.1.4.2, 9.1.5,9.1.6.1, 9.1.6.2 and Subsections 9.2 and 9.3, herein.

9.1.4.1	If an emergency type e-Tag is submitted and the Sink BA approves the e-Tag, WAPA may approve the transaction as the Source BA.

## 9.1.5	The Scheduling Entity will use WAPA DSW’s Open Access Same-Time Information System (OASIS) to create and submit a Transmission Service Request (TSR) in the amount of seasonal CROD rounded up to the nearest whole MW for each month of the Contract term.  The seasonal CROD, Point of Receipt (POR), and Point of Delivery (POD) for the transmission reservation are provided on Exhibit A of the Contract.  On the TSR, the Scheduling Entity shall include a Customer comment to show the Contractor’s unique contract number in this format, “PDP FES 28-DSR-#####.”  WAPA’s Transmission Business Unit will review and confirm the TSR through OASIS.  The TSR will be used on e-Tags for Quarterly Energy, Excess Energy, any elected Optional Energy, and COCP schedules.

## 9.1.6	The Scheduling Entity will pre-schedule Quarterly Energy, Excess Energy, any elected Optional Energy, and COCP schedules on a daily and hourly basis as follows:

9.1.6.1	Contractor will submit normal type e-Tags for its static schedule and adhere to WALC Business Practices, NAESB, NERC, WECC, and FERC requirements for normal type electronic schedules.  Specific WAPA e-Tag requirements are provided in Subsection 9.2, herein.  Daily pre-schedules will be subject to the concurrence of the WAPA Desert Southwest Region’s Energy Management and Marketing Office (EMMO), and the EMMO reserves the right to request adjustment to hourly schedules to comply with the Contract and this SABP.

9.1.6.2 The hourly scheduled energy for each Scheduling Entity will be determined by the pre-scheduled tagged amount as of 57 minutes prior to the Operating Hour.  Any change to the tagged energy after that time will be subject to settlement charges for Energy Imbalance as described further in Subsection 13.1.1, herein.

9.1.6.3 The daily sum of tagged energy for P-DP schedules in MST will be used by WAPA to assure that energy deliveries conform to Contractor’s Quarterly Energy, Excess Energy and any elected Optional Energy monthly amounts.

9.1.6.4 The monthly sum of tagged energy for P-DP Schedules in MST will be the basis to determine any overschedules by Contractor in Subsection 13.1.1.2 herein.

# 9.2	E-Tag Requirements for Schedules for Quarterly, Excess and Optional Energy:

## The Scheduling Entity will submit e-Tags for its schedules and adhere to WALC Business Practices, NAESB, NERC, WECC, and FERC requirements for schedules.  Per e-Tag specifications, all e-Tags will have whole megawatt values and contain the following:

## 9.2.1	DSWM01 shall be the first Purchasing Selling Entity (PSE) listed in the Market Path.

## 9.2.2	WALC shall be the Source Balancing Area (BA) with DSWM01 as the source PSE, and DSWM01 as the Generating Source in the Physical Path.

## 9.2.3	The e-Tag first Physical Segment Point of Receipt (POR) shall be stated in Exhibit A of the Contract.

9.2.4	WAPA will provide firm transmission to schedule from the POR to the Point of Delivery (POD) listed in Exhibit A of the Contract.

# 9.3	E-Tag Requirements for COCP Schedules:

## 9.3.1	The Scheduling Entity will submit e-Tags for its schedules and adhere to NAESB, NERC, WECC, and FERC requirements for schedules.  Per e-Tag specifications, all e-Tags will have whole megawatt values.

## 9.3.2	The Scheduling Entity may use Contractor’s P-DP FES TSR to schedule COCP resources on the existing TSR physical path.

## 9.3.3	The Scheduling Entity may use the OASIS to request an alternate transmission path for COCP resource scheduling.

9.3.3.1  Contractors with a Point-To-Point Transmission Service type reservation may request a redirect against the FES TSR.

9.3.3.1.a  Resale of redirect transmission capacity is prohibited.

9.3.3.2  Contractors with a Network Integration Transmission Service type reservation may request hourly non-designated service against the FES TSR.

# 9.3.4	WALC shall be the Transmission Service Provider for the physical path to the Contractor’s POD.

# 9.3.5	The Scheduling Entity shall not exceed rounded Hourly CROD published on the P-DP Application, as described in Section 10 herein, except as identified below:

# 9.3.5.1	The Contractor may elect Seasonal Full Transmission Usage for COCP resources scheduling, as described in Subsection 12.1, herein.

# 10.	HOURLY CROD:

10.1	For Quarterly Energy, Excess Energy, Optional Energy and COCP, WAPA will round CROD into whole megawatt portions for Contractor’s hourly scheduling purposes. WAPA will publish the rounded CROD available to schedule for every hour of the month, 15 days prior to the start of the month on the P-DP Application.  The Contractor’s aggregated Quarterly Energy, Excess Energy, Optional Energy, and COCP schedules, shall not exceed Contractor’s rounded Hourly CROD.

# 11.	GENERAL TRANSMISSION:

11.1	DSW merchant will coordinate with each Contractor’s SE to submit a 20-year TSR through the OASIS to match the Contractor’s rounded up P-DP FES CROD for Summer and Winter season.  Prior to October 1, 2028, all Contractors will request their TSR. Information about seasonal CROD, POR, and POD for the TSR will be provided on Exhibit A of the Contract.

11.2	Contractor’s SE will use their confirmed TSR to schedule Quarterly Energy, Excess Energy, Optional Energy, and COCP when using the same reserved transmission path.

11.3	Contractor’s SE may use the OASIS to request an alternate transmission path for scheduling of COCP, in accordance with Subsection 9.3.3.1 and 9.3.3.2, herein.

11.4	Contractor’s aggregated Quarterly Energy, Excess Energy, Optional Energy, and COCP schedule may not exceed their TSR in any hour.

11.5	In the event of transmission outages, Contractor’s SE will redirect the TSR to an alternate transmission path and reschedule their P-DP resources to best fit their portfolio needs.

- SEASONAL FULL TRANSMISSION USAGE FOR COCP:

  - Transmission is reserved, scheduled, and tracked in whole megawatts.   Contractor can choose to maximize receipt of COCP resources by requesting through the P-DP Application its Seasonal Full Transmission Usage in accordance with CROD values, as identified in Exhibit A of the Contract.  Contractor will be allowed to utilize the full TSR capacity associated with the Contract, consistent with existing procedures available to transmission customers.

    - Contractors may elect Seasonal Full Transmission Usage for the month in the P-DP Application.  Contractors must elect Seasonal Full Transmission Usage 45 days prior to the beginning of the month to allow for advance billing.  Contractors will be billed in accordance with Subsection 13.4, herein.

- POWER ACCOUNTING AND BILLING PROCEDURES:

# 13.1	Quarterly Energy Billing and Accounting:

## 13.1.1	Quarterly Energy shall be billed in advance in accordance with Section 13 of the Contract.  The number of kilowatt hours (kWh) advanced billed shall be the kWh energy posted in the P-DP Application for the month.  Minimal differences between scheduled amounts and the amounts posted in the P-DP Application due to rounding of schedules to whole MW are expected.  No billing adjustments for these variances will occur, and the Quarterly Energy shall be billed at the amount(s) shown on the P-DP Application.

13.1.1.1 If Contractor underschedules its Quarterly Energy, the Scheduling Entity will coordinate with EMMO to arrange for the energy to be scheduled in a future month(s).

13.1.1.2 If Contractor overschedules its Quarterly Energy, at WAPA’s discretion, either the Scheduling Entity will coordinate with EMMO to arrange for a reduction in their quarterly energy within two months from the month in which the overschedule occurred or the Contractor will be billed for the overscheduled amount at the final Optional Energy price for that month.

# 13.2	Excess Energy Distribution and Accounting Process:

## 13.2.1	When Excess Energy is deemed to be available using the calculation in Subsection 8.3.1, herein, Excess Energy will be offered to all Contractors and PUP recipients based on a pro rata share of their seasonal CROD.  Reclamation may update hydro generation forecasts on a frequent basis and will provide such information to WAPA.  Excess Energy distributed in part of a year may be subject to adjustment in subsequent months if the yearly generation projection drops below the Excess Energy threshold later that year.

## 13.2.2	Excess Energy will be billed in accordance with the applicable P-DP rate schedule as contemplated in the Contract and included on Contractor’s invoice as a separate advance payment line item.

# 13.3	Optional Energy Billing and Accounting:

13.3.1	Sixty days in advance of each month, WAPA will post in the P-DP Application an estimated price for Optional Energy.  Contractors may elect Optional Energy for the month in the P-DP Application by identifying its elected kWh amount and desired hours.  Contractor must elect Optional Energy 45 days prior to the beginning of the month to allow for advance billing.  Contractor’s elected kWh amount of Optional Energy multiplied by the estimated price will be billed on the FES invoice as a separate advance payment line item.

13.3.2	At the end of the month WAPA will calculate the amount of actual Optional Energy purchased, multiplied by the actual average price, and will either debit or credit the Contractor’s FES invoice in a future billing cycle.  WAPA reserves the right to send out a supplemental invoice for advance funding in the event purchases are estimated to exceed price and quantity of Optional Energy requested.

13.3.3	The cost of providing Optional Energy shall not be included in the P-DP FES Rate Schedule.

13.4	Transmission for COCP:

13.4.1 Contractor electing Seasonal Full Transmission Usage of their TSR will be billed at the existing transmission service rate applicable for the P-DP, for the rounded up seasonal CROD value for each month. Contractor’s elected kW amount of Seasonal Full Transmission Usage minus their CROD kW provided for in Exhibit A of the Contract, will be multiplied by the existing transmission service rate applicable to P-DP and will be billed on the FES invoice as a separate advance payment line item.

13.4.1	If Seasonal Full Transmission Usage is not elected and advanced billed, and the transmission is used, the additional transmission usage will be billed in a future billing cycle on the FES invoice as a separate line item..

13.5	Transmission Losses:

13.5.1	WAPA is responsible for transmission line losses to the POD on the P-DP system designated in Exhibit A of the Contract, and such losses will be included in the applicable P-DP rate schedule as contemplated in the Contract for Quarterly Energy and Excess Energy.  No losses will be charged on Quarterly Energy and Excess Energy deliveries to the POD on WAPA’s transmission system designated in Exhibit A of the Contract.

13.5.2	Intentionally omitted

13.5.3 Any transmission losses on deliveries for Optional Energy that Contractor is responsible for under the Contract will be calculated using the monthly average loss rate for the month.

13.5.4 Any transmission losses on deliveries of COCP that Contractor is responsible for under the Contract will be billed to the Contractor in accordance with the applicable WAPA business practices and Transmission Losses Service rate schedule.

# 13.6	Energy Imbalance Market (EIM) Settlements:  All applicable EIM settlements will be allocated per WAPA DSW’s Energy Imbalance Market Business Practice.